Development Tools Assignment

Module 2 – Network Performance Measurement Tools

- Install Wireshark, take capture on WiFi interface

- Identify the beacon frame using filter.

- Answer:

- To capture beacon frames “wlan.fc.type_subtype == 0x08” is the filter type.

- beacons are part of the management frames which has the type field set to 0 and beacons are represented by a hex value of 0x8.

- (there are no beacon frames because I don’t have wifi moniter mode support).

- Apply filters to view specific packet.

- Answer:

- Type “tcp” in filter to view only tcp protocol packets

Type “udp” in filter to view only udp protocol packets

- Decrypt the wireshark pcap using passphrase, to view the encrytped packets.

- Answer:

- The passphrase is the password of the WiFi Router.

- Point out ethernet and 802.11 frames.

- Answer:

- Type “eth” in filter to show only frames sent/received over ethernet.

- Type “wlan” in filter to show only 802.11 frames.

2) Install Iperf on client and server device

1. Run TCP traffic

> client side: iperf3 -c <ip-address>

> server side: iperf3 -s

Average Transfer rate : 32.0 GBytes

Average Bitrate : 26.4 Gbits/sec

- Run UDP traffic

> client side: iperf3 -c <ip-address> -u

> server side: iperf3 -s

Average Transfer rate : 1.25 Mbytes

Average Bitrate : 1.04 Mbits/sec

Jitter : 0.036 ms

Lost/total Datagrams : 0/40 (0%)